TECHNICAL ANNEX

Hanna Statistics — Specifications & Data Analysis

Reference document for the Development Proposal  |  March 2026  |  Rev. 1.2

# 1. Operational Flow

## 1.1 Software Ecosystem

Three applications are currently installed on dedicated Windows workstations at the Nusfalau plant:

| Software | Context | Users |
| --- | --- | --- |
| Chemical Production | Production lines — reagents (powder/tablet) | Production technicians |
| Chemical QC | QC chemistry laboratory | QC operators |
| Chemical MR | Production of pH buffer solutions used as calibration standards by Chemical QC | Buffer production technicians |

All three applications share the product code database (dbCode.mdb). Chemical Production communicates with Chemical QC via QR code labels attached to physical samples.

## 1.2 Chemical Production Pipeline

- Recipe formulation (raw materials, quantities, reagent sets)

- Batch Preparation → exports PREP .xlsx file + prints Brother labels (lot number, QR code)

- Prints QR code "to QC" → physical label that accompanies the sample to the QC lab

- Iterative QC cycle: if QC fails, sample returns to production for correction (Preparation QC module records correction date, CM used, grams added)

- On QC pass: scan QR "QC closed" → lot closed, Hanna Code exported to dbCode.mdb

Chemical Production handles powder and tablet reagents (Hanna Code products). The exported PREP file for each batch contains preparation details, component weights and deviations.

## 1.3 Chemical QC Pipeline

- Receives sample from production (scan QR "to QC")

- Information QC: lot metadata entry/verification

- Reading QC: instrumental readings for each STD (up to 4 meters M1–M4, MIN/MAX per standard). CORRECTION column handles individual outlier exclusion.

- Evaluation QC: valid reading selection (≥70–80%), mean calculation, final lot verdict (Passed/Failed)

- Export .xlsx per lot → QC lot file saved locally (naming: CP-R{n}_{HannaCode}_{STD}_LOT{n}_PWW{week}_{year}_QC.xlsx)

This exported .xlsx file — one per lot — is the primary data source for Module 1.

## 1.4 Chemical MR Pipeline

Chemical MR manages the production of pH buffer solutions (pH 1.68, 4.01, 7.01, 10.01). These buffers are used by Chemical QC as calibration standards when testing Hanna Code reagent lots produced by Chemical Production. Chemical MR also handles the manual preparation of reference standards used in the QC process.

Chemical MR exports one PREP .xlsx file per buffer lot, containing preparation details, correction data (CM code, quantity added), and production quantities. These files are the primary data source for Module 2 (Buffer Production Statistics) and Module 3 (Preparation Statistics).

## 1.5 How Hanna Statistics fits in

Hanna Statistics sits downstream from all three applications. It reads the .xlsx files already exported by Chemical QC, Chemical Production and Chemical MR — one file per lot — and does not interfere with their operation in any way.

The two documents currently maintained manually by the team are what Hanna Statistics replaces:

| Module | Data source | Replaces |
| --- | --- | --- |
| Module 1 — Reagent QC | Chemical QC — one .xlsx file per lot | Statistic_HI782-0.xlsx and one equivalent per product code |
| Module 2 — Buffer Production | Chemical MR — one .xlsx file per buffer lot Reads: production quantities, corrections, Variance fields | Buffer Preparation and Correction Statistic.xlsx — plus Statistic_L84_Preparation_List (source to confirm) |
| Module 3 — Preparation | Chemical Production + Chemical MR — all PREP .xlsx files Reads: operator, planned vs actual week, component traceability | No equivalent today — new capability |

Note: Module 2 and Module 3 both read Chemical MR files, but extract different information. Module 2 reads production and correction data (quantities, Variance, CM used) to track buffer quality. Module 3 reads preparation details (operator, planned vs actual week, components) for traceability. The files are the same — the questions they answer are different.

# 2. Real Data Analysis

Before defining requirements, the documents currently in use at the plant were analyzed to understand what data is available, what calculations are performed today, and what Hanna Statistics needs to reproduce — and improve upon.

## 2.1 QC Statistics Workbook: Statistic_HI782-0

Product: HI782-0 — Marine Nitrate HR Reagent (CP-R80, Line L57 Powder)

This workbook is manually maintained by the QC team. For each new lot, the operator imports the Chemical QC export file via Power Query, verifies meter column alignment, and recalculates the control chart.

| Parameter | Value |
| --- | --- |
| STD values | 4 standards: 0, 15, 35, 60 ppm |
| Sigma per STD | σ(0)=1.00, σ(15)=1.3675, σ(35)=1.8575, σ(60)=2.47 |
| Number of lots | 25 lots (LOT0366 → LOT0972) |
| Period | January 2024 – March 2025 |
| Total readings | 2,956 readings |
| Avg readings per lot | ~118 readings/lot (range: 36–295) |
| TEST types found | ASTM, ASTM D665, EPP:170–250mg, Old A–D, P/Final, P/Prod, Valid |
| Excel structure | 5 sheets; STD All sheet = 133 columns |

Critical observations:

- Sigma bands are calculated as absolute values per STD — not statistical standard deviations. Running averages are cumulative lot-by-lot.

- The STD All sheet extends to 133 columns: filter selectors (1–10), raw readings with means per lot (51–58), running averages and sigma limits per lot per STD (60–133).

What the current Excel workbook shows — and the question each view answers:

| What they see today | The question it answers |
| --- | --- |
| Control chart — raw readings scattered per lot | Is this lot's quality consistent, or are the readings all over the place? |
| Control chart — running average line | Is the product's quality improving, stable, or slowly drifting lot after lot? |
| Sigma bands (1σ / 2σ / 3σ) | How close are we to the tolerance limit? Are we comfortable in the green zone, or creeping toward yellow? |
| Sigma distribution table (% within 1σ / 2σ / 3σ) | For this specific lot, how many readings were well within tolerance, how many were marginal, how many were a problem? |

## 2.2 Buffer Statistics Documents

Two documents are currently maintained manually by the production team for buffer lot tracking:

- Buffer Preparation and Correction Statistic 2024–2025.xlsx — aggregated correction statistics across all four pH values (C158/1.68, C163/4.01, C164/7.01, C166/10.01). Fed by Chemical MR lot exports.

- Statistic_L84_Preparation_List_2024_01_01-2024_12_31_pH.pdf — annual preparation list per recipe (e.g. C166 / pH 10.01), showing all lots produced during the year with lot number, preparation date, preparation week, corrections applied (CM code + grams), QC date, and quantity produced. Source software to confirm — see Section 6.3.

Hanna Statistics calculates the same information — and more — automatically from the individual Chemical MR lot files, without manual aggregation.

| Sheet | Recipe | Lots | Corrections | Notes |
| --- | --- | --- | --- | --- |
| pH 1.68 | C158 | 18 | 6 | Avg ~350 Kg/lot |
| pH 4.01 | C163 | 175 | 54 (31%) | Highest volume |
| pH 7.01 | C164 | 168 | 33 (20%) |  |
| pH 10.01 | C166 | 88 | 44 (50%) | Most critical |
| TOTAL |  | 449 | 137 | Period: Jan 2024 – Sep 2025 |

8 Correction Materials (CM):

CM011 (Potassium Tetraoxalate), CM021 (Potassium Hydrogen Phthalate), CM131 (Hiamine), CM480 (KOH 90%), CM022 (KH₂PO₄), CM023 (Na₂HPO₄), CM029 (Na₂CO₃), CM030 (NaHCO₃).

What the current documents show — and the question each view answers:

| What they see today | The question it answers |
| --- | --- |
| Production quantity per lot | How much buffer did we produce, and when? |
| FIRST QC FAILED value | When the lot failed, how far off was the pH reading from the target? |
| CM USED FOR CORRECTION | What did we add to fix it? |
| corr CM [g] / corr CM [%] | How much did we have to add — was it a small adjustment or a large one? |
| Corr Gibertini [%] | How far was the batch from the target recipe before any correction? |
| Monthly correction count | In which months do we correct more lots? |

Note: In some cases, correction of the same lot requires two different CM components. This is handled in the data model. Does corrections come from Variance fields in excel files?

## 2.3 Estimated Annual Volumes

| Source | Est. Volume/Year |
| --- | --- |
| Chemical QC lot files (.xlsx) | ~1,000–1,100 files |
| Chemical Production PREP files (.xlsx) | ~650–700 files |
| Chemical MR PREP files (.xlsx) | to confirm |
| Estimated QC readings | ~120,000 new readings/year |
| Distinct product codes (sample set) | 99 Hanna Codes |

# 3. Functional Requirements

Source: Procedure Statistic Analysis QC readings — Rev. 0.0 Draft, Hanna Instruments SRL Nusfalau

## 3.1 Scope

The system manages statistics for powder reagents from the QC chemistry laboratory. Objectives: visualize lot-to-lot mean evolution trends, verify preparation precision and repeatability, and evaluate each lot quality against Hanna tolerances.

## 3.2 Key Definitions

| Term | Definition |
| --- | --- |
| STD | Reference Standard value used for calibration |
| M1–M4 | Meter 1–4: measurement instruments used during QC |
| σ | 50% of Hanna Tolerance for each STD value (FIXED — NOT a statistical standard deviation) |
| P/Final | Test type: final production measurement |
| P/Prod | Test type: intermediate production measurement |

## 3.3 Control Chart Requirements

The main chart displays 3 overlaid series for each selected STD:

| Series | Chart Type | Description |
| --- | --- | --- |
| Raw readings | Scatter plot | All data points per lot, numbered sequentially, columns separated per lot |
| Lot-by-lot means | Scatter + line | Running average cumulative lot-by-lot |
| Sigma bands | Stacked area | 1σ green, 2σ blue, 3σ yellow (centered on STD value) |

Sigma band calculation:

- 1σ = STD Value ± (50% × Hanna Tolerance) → green

- 2σ = STD Value ± (100% × Hanna Tolerance) → blue

- 3σ = STD Value ± (150% × Hanna Tolerance) → yellow

Each STD has its own bands (tolerance varies by STD value).

## 3.4 Statistical Table Requirements

Per lot, per STD: lot number, QC date, QC week, grand total tests, total tests per STD, then count and % of readings within <±1σ, ±1σ–±2σ, ±2σ–±3σ, >±3σ.

## 3.5 Filters

- Hanna Code (product selection)

- TEST type — confirmed types for standard analysis: P/Final and P/Prod. The lot files also contain ASTM, ASTM D665, EPP variants, Old A–D, and Valid. All are supported and selectable as filters, but not included in the default analysis view.

- STD values: up to 6 standards per product, selectable in ascending order

- Date range

# 4. Architecture

This section describes the technologies proposed to build Hanna Statistics. It is included here for completeness and for any technical review, but it does not affect the functional scope of the software.

In practical terms: the application uses a standard web technology stack — the same approach used by modern industrial software — with a dedicated database that stores all imported data in a structured, queryable format. The database runs locally on the Statistics PC in V1, with no external dependencies. If the system is later moved to a server for multi-user access (V2), the same database migrates without any changes to its structure or to the application itself.

## V1 and V2 — How the system can grow

V1 — One PC, everything works. Hanna Statistics is installed on a dedicated PC — typically the QC lab manager's or production manager's workstation. The .xlsx files exported by the three Gibertini applications are picked up automatically via the shared network folder, and the software imports them, processes them and displays the results. No server, no complex network configuration. Install and use — exactly like the other Gibertini applications.

V2 — The same data, accessible from any PC on the network (optional). If in the future you want to access the statistics from multiple workstations — for example from both the QC lab and the production office — there is no need to rewrite the software. It would simply be a matter of moving the database to a server already present at the plant (e.g. the existing one at 192.168.1.36) and opening a browser on any PC on the internal network. Same software, same database, same interface — accessible from multiple points simultaneously. This path is already built into the architecture: it is not a hypothetical future option, it is a configuration change.

Note: Hanna has asked whether it is possible to start directly with V2 (multi-user, server-based). This is technically feasible and will be discussed with the IT team — see Section 5.

## 4.1 Technology Stack

| Layer | Technology |
| --- | --- |
| User Interface | React 19 + TypeScript + Vite |
| UI Components | shadcn/ui + Tailwind CSS 4 |
| Charts | Apache ECharts |
| Tables | TanStack Table v8 (server-side pagination) |
| Backend / API | Node.js 20 + Express |
| Database | MariaDB 10.11+ |
| Network access | Nginx reverse proxy + HTTPS self-signed (V2 only) |
| Deployment | Docker Compose |
| Languages | English and Romanian (V1) — Italian and Hungarian available on request |

# 5. Open Questions

This section summarises the status of all questions. Confirmed answers are in green. Questions for the IT team call are in red. Questions still awaiting Hanna's input are in amber.

## 5.1 Confirmed

Confirmed: Languages: English and Romanian

Confirmed: TEST types for standard analysis: P/Final and P/Prod. All other types present in the lot files (ASTM, EPP, Old, Valid) are supported and selectable as filters, but not included in the default view.

Confirmed: Users: both QC lab manager and production manager.

Confirmed: Data retention: 10 years.

Confirmed: Hanna Core integration: confirmed as a desired future path, as a module on the existing server (192.168.1.36) alongside Hanna Core RecordBook.

Confirmed: Network: all PCs at the plant are already connected to the same local network. Automatic file import via shared network folder is possible without additional infrastructure.

Confirmed: Multiple CM corrections per lot: in some cases a buffer lot requires correction with two or more? different CM components. Noted and handled in the data model.

## 5.2 To Discuss — Call with IT Team

To discuss with IT team: Deployment: start with V1 (one PC, local database) or directly with V2 (server-based, accessible from multiple PCs via browser)? Both are feasible — depends on available server infrastructure.

To discuss with IT team: Network folder configuration: which folder path will be used as the shared import location? Which PCs need read/write access?

## 5.3 Awaiting Hanna's Input

Note: Statistic_L84_Preparation_List source — which software generates this document? Chemical MR, Chemical Production, or is it a manual report?

Note: Scope of the first delivery: what do you want to be able to see and use at the end of Phase 1 — only the Reagent QC Statistics (control chart and sigma distribution table), or also the Buffer Production module?

Note: Missing analyses: are there questions about your production process that you currently cannot answer from the existing documents — things you would like to see but cannot today?