TECHNICAL ANNEX

Hanna Statistics — Specifications & Data Analysis

Reference document for the Development Proposal  |  March 2026  |  Rev. 0

# 1. Operational Flow

## 1.1 Software Ecosystem

Three applications are currently installed on dedicated Windows workstations at the Nusfalau plant:

| Software | Context | Users |
| --- | --- | --- |
| Chemical Production | Production lines — reagents (powder/tablet) | Production technicians |
| Chemical QC | QC chemistry laboratory | QC operators |
| Chemical MR | Buffer production (pH solutions) | Buffer production technicians |

All three applications share the product code database (dbCode.mdb). Chemical Production and Chemical MR communicate with Chemical QC via QR code labels attached to physical samples.

## 1.2 Chemical Production Pipeline

- Recipe formulation (raw materials, quantities, reagent sets)

- Batch Preparation → exports PREP .xlsx file (pattern: PREP_CP-B{n}.L56.CTK.*) + prints Brother labels (lot number, QR code)

- Prints QR code "to QC" → physical label that accompanies the sample to the QC lab

- Iterative QC cycle: if QC fails, sample returns to production for correction (Preparation QC module records correction date, CM used, grams added)

- On QC pass: scan QR "QC closed" → lot closed, Hanna Code exported to dbCode.mdb

## 1.3 Chemical QC Pipeline

- Receives sample from production (scan QR "to QC")

- Information QC: lot metadata entry/verification

- Reading QC: instrumental readings for each STD (up to 4 meters M1–M4, MIN/MAX per standard). CORRECTION column handles individual outlier exclusion.

- Evaluation QC: valid reading selection (≥70–80%), mean calculation, final lot verdict (Passed/Failed)

- Export .xlsx → QC lot file saved locally (naming: CP-R{n}_{HannaCode}_{STD}_LOT{n}_PWW{week}_{year}_QC.xlsx)

This exported .xlsx file is the primary data source for Module 1 (Reagent QC Statistics).

## 1.4 Chemical MR Pipeline

Chemical MR manages the production of STD <buffer solutions> (pH 1.68, 4.01, 7.01, 10.01).

- Recipe formulation (raw materials, quantities, density)

- Batch Preparation → exports PREP .xlsx files (patterns: PREP_B{n}-SOL.A/B.*, PREP_B{n}-A/B.*, PREP_{HannaCode}.MR{n}.*)

- Export .xlsx per buffer lot → contains a table with per-standard deviations (MR Qty, MR Acquired, Variance, Variance Perc). This data feeds the Buffer Statistic aggregation file.

The PREP files exported by Chemical MR are the primary source for Module 2 (Buffer Production Statistics) and Module 3 (Preparation Statistics).

## 1.5 Data Flow Between Systems

The three applications exchange data through:

- Physical labels: QR "to QC" (production → lab) and QR "QC closed" (lab → production)

- Excel exports: PREP .xlsx (Chemical Production and Chemical MR), QC .xlsx (Chemical QC), Buffer Statistic .xlsx (aggregated from Chemical MR data)

Hanna Statistics sits downstream from all three applications: it imports the .xlsx files already produced by Chemical QC, Chemical MR and Chemical Production, without interfering with their operation.

# 2. Real Data Analysis

Before defining requirements, actual Excel files from the plant were analyzed to understand data structure, volumes, and complexity.

## 2.1 QC Statistics File: HI782-0

Product: HI782-0 — Marine Nitrate HR Reagent (CP-R80, Line L57 Powder)

| Parameter | Value |
| --- | --- |
| STD values | 4 standards: 0, 15, 35, 60 ppm |
| Sigma per STD | σ(0)=1.00, σ(15)=1.3675, σ(35)=1.8575, σ(60)=2.47 |
| Number of lots | 25 lots (LOT0366 → LOT0972) |
| Period | January 2024 – March 2025 |
| Total readings | 2,956 readings |
| Avg readings per lot | ~118 readings/lot (range: 36–295) |
| TEST types found | ASTM, ASTM D665, EPP:170–250mg, Old A–D, P/Final, P/Prod, Valid |
| Excel structure | 5 sheets; STD All sheet = 133 columns |

Observations:

- Sigma bands are calculated as absolute values per STD (not statistical standard deviations). Running averages are cumulative lot-by-lot.

- The STD All sheet extends to 133 columns: filter selectors (1–10), raw readings with means per lot (51–58), running averages and sigma limits per lot per STD (60–133).

| What i see in Excel | The question it answers |
| --- | --- |
| Control chart — raw readings scattered per lot | Is this lot's quality consistent, or are the readings all over the place? |
| Control chart — running average line | Is the product's quality improving, stable, or slowly drifting lot after lot? |
| Sigma bands (1σ / 2σ / 3σ) | How close are we to the tolerance limit? Are we comfortable in the green zone, or creeping toward yellow? |
| Sigma distribution table (% within 1σ / 2σ / 3σ) | For this specific lot, how many readings were well within tolerance, how many were marginal, how many were a problem? |

## 2.2 Buffer Statistic File

Coverage: Buffer Preparation and Correction Statistic 2024–2025

This file is an aggregation of data exported by Chemical MR. It is maintained manually as a summary workbook fed by individual Chemical MR lot exports.

| Sheet | Recipe | Lots | Corrections | Notes |
| --- | --- | --- | --- | --- |
| pH 1.68 | C158 | 18 | 6 | Avg ~350 Kg/lot |
| pH 4.01 | C163 | 175 | 54 (31%) | Highest volume |
| pH 7.01 | C164 | 168 | 33 (20%) |  |
| pH 10.01 | C166 | 88 | 44 (50%) | Most critical |
| TOTAL |  | 449 | 137 | Period: Jan 2024 – Sep 2025 |

Note: corr CM [g] ← Variance (absolute deviation, grams); corr CM [%] / Corr Gibertini [%] ← Variance Perc (% deviation) from the Chemical MR per-lot export table. The exact mapping needs confirmation — see Section 7.2, Question 8.

| What i see in Excel | The question it answers |
| --- | --- |
| Production quantity per lot | How much buffer did we produce, and when? |
| FIRST QC FAILED value | When the lot failed, how far off was the pH reading from the target? |
| CM USED FOR CORRECTION | What did we add to fix it? |
| corr CM [g] / corr CM [%] | How much did we have to add |
| Corr. [%] | How far was the batch from the target recipe before any correction? |

## 2.3 Estimated Annual Volumes

| Source | Est. Volume/Year |
| --- | --- |
| Chemical QC lot files (.xlsx) | ~1,000–1,100 files |
| Chemical Production + Chemical MR PREP files (.xlsx) | ~650–700 files |
| Total files to manage | ~1,700–1,800 files/year |
| Estimated QC readings | ~120,000 new readings/year |
| Distinct product codes (sample set) | 99 Hanna Codes |

# 3. Functional Requirements

Source: Procedure Statistic Analysis QC readings — Rev. 0.0 Draft, Hanna Instruments SRL Nusfalau

## 3.1 Scope

The system manages statistics for powder reagents from the QC chemistry laboratory. Objectives: visualize lot-to-lot mean evolution trends, verify preparation precision and repeatability, and evaluate each lot quality against Hanna tolerances.

## 3.2 Key Definitions

| Term | Definition |
| --- | --- |
| STD | Reference Standard value used for calibration |
| M1–M4 | Meter 1–4: measurement instruments used during QC |
| σ | 50% of Hanna Tolerance for each STD value |
| P/Final | Test type: final production measurement |
| P/Prod | Test type: intermediate production measurement |

## 3.3 Control Chart Requirements

The main chart displays 3 overlaid series for each selected STD:

| Series | Chart Type | Description |
| --- | --- | --- |
| Raw readings | Scatter plot | All data points per lot, numbered sequentially, columns separated per lot |
| Lot-by-lot means | Scatter + line | Running average cumulative lot-by-lot |
| Sigma bands | Stacked area | 1σ green, 2σ blue, 3σ yellow (centered on STD value) |

Sigma band calculation:

- 1σ = STD Value ± (50% × Hanna Tolerance) → green

- 2σ = STD Value ± (100% × Hanna Tolerance) → blue

- 3σ = STD Value ± (150% × Hanna Tolerance) → yellow

Each STD has its own bands (tolerance varies by STD value).

## 3.4 Statistical Table Requirements

Per lot, per STD: lot number, QC date, QC week, grand total tests, total tests per STD, then count and % of readings within <±1σ, ±1σ–±2σ, ±2σ–±3σ, >±3σ.

## 3.5 Filters

- Hanna Code (product selection)

- TEST type (P/Final, P/Prod, ASTM, EPP:xxx, Old, Valid…)

- STD values: up to 6 standards per product, selectable in ascending order

- Date range

# 4. Architecture

This section describes the technologies proposed to build Hanna Statistics. It is included here for completeness and for any technical review, but it does not affect the functional scope of the software.

In practical terms: the application uses a standard web technology stack — the same approach used by modern industrial software — with a dedicated database that stores all imported data in a structured, queryable format. The database runs locally on the Statistics PC in V1, with no external dependencies. If the system is later moved to a server for multi-user access (V2), the same database migrates without any changes to its structure or to the application itself.

## 4.1 Technology Stack

| Layer | Technology |
| --- | --- |
| User Interface | React 19 + TypeScript + Vite |
| UI Components | shadcn/ui + Tailwind CSS 4 |
| Charts | Apache ECharts |
| Tables | TanStack Table v8 (server-side pagination) |
| Backend / API | Node.js 20 + Express |
| Database | MariaDB 10.11+ |
| Network access | Nginx reverse proxy + HTTPS self-signed (V2 only) |
| Deployment | Docker Compose |
| Languages | EN, IT, RO, HU |

V1 (Desktop): MariaDB installed locally on Windows (Windows service, no server required). V2 (Server): same MariaDB instance migrated to Linux LAN server — no schema changes needed.

V1 / V2 — System Evolution

How the Gibertini software works today — and how Hanna Statistics will work

Chemical Production, Chemical QC and Chemical MR are installed on dedicated fixed workstations. Each application runs on one machine, each machine is in its department. Hanna Statistics will work exactly the same way in its initial version: a program installed on a PC, used by whoever needs it, with no dependency on internet connectivity or servers.

V1 — One PC, everything works Hanna Statistics is installed on a dedicated PC — typically the QC lab manager's or production manager's workstation. The .xlsx files exported by Chemical QC, Chemical MR and Chemical Production are copied to that PC (manually via USB, or automatically if a shared network folder already exists), and the software imports them, processes them and displays the results. No server, no complex network configuration. Install and use — exactly like the other Gibertini applications.

V2 — The same data, accessible from any PC on the network (optional) If in the future you wanted to access the statistics from multiple workstations — for example from both the QC lab and the production office — there would be no need to rewrite the software. It would simply be a matter of moving the database to a server already present at the plant (e.g. the existing one at 192.168.1.36) and opening a browser on any PC on the internal network. Same software, same database, same interface — accessible from multiple points simultaneously. This path is already built into the architecture proposed: it is not a hypothetical future option, it is a configuration change.

Why not extend the existing software?

Chemical Production, Chemical QC and Chemical MR were built for a specific purpose: managing individual lots in real time, one operation at a time.

Statistical analysis is a fundamentally different kind of work. It means holding thousands of readings in memory simultaneously, calculating running averages across dozens of lots, drawing charts that span months of production, and answering questions that cut across multiple products at once. The architecture of the existing applications — built on VB6 and MS Access, one record at a time, one user at a time — was never designed for that.

Adding a full statistics module on top of them would mean rebuilding large parts of three critical production applications, introducing risk into software that currently runs in production and laboratory on many stations, for a result that would still be constrained by the same architectural limits.

A dedicated application, with a dedicated database built for analytical queries, is the right tool for the job.

# 5. Open Technical Questions

Questions are grouped into blocking (must be resolved before development starts) and development-phase (can be resolved during implementation).

- V1 Scope: Reagent QC only, or also Buffer Production and Preparation?

- Number of workstations: 1 (standalone, local DB) or 2+ (requires data synchronization)?

- Shared network folder: accessible from production, QC lab, and Statistics PC?

The three Gibertini applications each save their export files on the PC where they are installed — the QC lab PC, the production PC. If all these machines are visible to each other on the plant's local network, Hanna Statistics can reach those files directly and import them automatically, without anyone having to manually copy anything.

If the PCs are not networked, or if it is simpler to keep them separate, a manual copy via USB works just as well. The import process is identical either way — only the step of moving the file to the Statistics PC changes.

- Test Type: The official requirements document mentions two test types: P/Final and P/Prod. The real lot files from the plant contain several more — ASTM, ASTM D665, EPP variants (170mg, 171mg, 172mg, 250mg), Old A–D, and Valid. The question is which ones should be selectable as filters in the interface, and whether any of them should be excluded from the standard analysis by default.

- Users: who will use Hanna Statistics? QC lab manager only? Production manager? Both?

- Languages for V1: Do you need multilanguage approach? If Yes, all four (EN/IT/RO/HU) or start with a subset?

- Data retention: how many years to keep raw readings?

- Future Hanna Core integration: is integration with Hanna Core RecordBook (as a module on the same server, 192.168.1.36) a desired path?

- Buffer Statistic file: Please confirm that in the Buffer Statistic file Corr [%] and corr CM [%] comes from the Chemical MR excel output files.

- Are there any statistics or analyses you currently feel are missing from the Excel workbooks — questions about your production process that you would like to answer but cannot today?