TECHNICAL ANNEX

Hanna Statistics — Specifications & Data Analysis

Reference document for the Development Proposal  |  March 2026  |  Rev. 1.1

# 1. Operational Flow

## 1.1 Software Ecosystem

Three applications are currently installed on dedicated Windows workstations at the Nusfalau plant:

| Software | Context | Users |
| --- | --- | --- |
| Chemical Production | Production lines — reagents (powder/tablet) | Production technicians |
| Chemical QC | QC chemistry laboratory | QC operators |
| Chemical MR | Buffer production (pH solutions) | Buffer production technicians |

All three applications share the product code database (dbCode.mdb). Chemical Production and Chemical MR communicate with Chemical QC via QR code labels attached to physical samples.

Chemical MR is an application dedicated to STD buffer solution production (pH 1.68, 4.01, 7.01, 10.01). The architecture: recipe management, formulations, Manual and STD preparations, and Excel export in the Chemical QC style.

## 1.2 Chemical Production Pipeline

- Recipe formulation (raw materials, quantities, reagent sets)

- Batch Preparation → exports PREP .xlsx file (pattern: PREP_CP-B{n}.L56.CTK.*) + prints Brother labels (lot number, QR code)

- Prints QR code "to QC" → physical label that accompanies the sample to the QC lab

- Iterative QC cycle: if QC fails, sample returns to production for correction (Preparation QC module records correction date, CM used, grams added)

- On QC pass: scan QR "QC closed" → lot closed, Hanna Code exported to dbCode.mdb

## 1.3 Chemical QC Pipeline

- Receives sample from production (scan QR "to QC")

- Information QC: lot metadata entry/verification

- Reading QC: instrumental readings for each STD (up to 4 meters M1–M4, MIN/MAX per standard). CORRECTION column handles individual outlier exclusion.

- Evaluation QC: valid reading selection (≥70–80%), mean calculation, final lot verdict (Passed/Failed)

- Export .xlsx → QC lot file saved locally (naming: CP-R{n}_{HannaCode}_{STD}_LOT{n}_PWW{week}_{year}_QC.xlsx)

This exported .xlsx file is the primary data source for Module 1 (Reagent QC Statistics).

## 1.4 Chemical MR Pipeline

Chemical MR manages the production of STD <buffer solutions> (pH 1.68, 4.01, 7.01, 10.01).

- Recipe formulation (raw materials, quantities, density)

- Batch Preparation → exports PREP .xlsx files (patterns: PREP_B{n}-SOL.A/B.*, PREP_B{n}-A/B.*, PREP_{HannaCode}.MR{n}.*)

- Export .xlsx per buffer lot → contains a table with per-standard deviations (MR Qty, MR Acquired, Variance, Variance Perc). This data feeds the Buffer Statistic aggregation file.

The PREP files exported by Chemical MR are the primary source for Module 2 (Buffer Production Statistics) and Module 3 (Preparation Statistics).

Note: Note: The exact internal structure of the Chemical MR per-lot export (table format, column names for Variance/Variance Perc) needs confirmation before implementing the Module 2 parser. See Section 7.2, Question 8.

## 1.5 Data Flow Between Systems

The three applications exchange data through:

- Physical labels: QR "to QC" (production → lab) and QR "QC closed" (lab → production)

- Excel exports: PREP .xlsx (Chemical Production and Chemical MR), QC .xlsx (Chemical QC), Buffer Statistic .xlsx (aggregated from Chemical MR data)

Hanna Statistics sits downstream from all three applications: it imports the .xlsx files already produced by Chemical QC, Chemical MR and Chemical Production, without interfering with their operation.

# 2. Real Data Analysis

Before defining requirements, actual Excel files from the plant were analyzed to understand data structure, volumes, and complexity.

## 2.1 QC Statistics File: HI782-0

Product: HI782-0 — Marine Nitrate HR Reagent (CP-R80, Line L57 Powder)

| Parameter | Value |
| --- | --- |
| STD values | 4 standards: 0, 15, 35, 60 ppm |
| Sigma per STD | σ(0)=1.00, σ(15)=1.3675, σ(35)=1.8575, σ(60)=2.47 |
| Number of lots | 25 lots (LOT0366 → LOT0972) |
| Period | January 2024 – March 2025 |
| Total readings | 2,956 readings |
| Avg readings per lot | ~118 readings/lot (range: 36–295) |
| TEST types found | ASTM, ASTM D665, EPP:170–250mg, Old A–D, P/Final, P/Prod, Valid |
| Excel structure | 5 sheets; STD All sheet = 133 columns |

Observations:

- Sigma bands are calculated as absolute values per STD (not statistical standard deviations). Running averages are cumulative lot-by-lot.

- The STD All sheet extends to 133 columns: filter selectors (1–10), raw readings with means per lot (51–58), running averages and sigma limits per lot per STD (60–133).

| What i see in Excel | The question it answers |
| --- | --- |
| Control chart — raw readings scattered per lot | Is this lot's quality consistent, or are the readings all over the place? |
| Control chart — running average line | Is the product's quality improving, stable, or slowly drifting lot after lot? |
| Sigma bands (1σ / 2σ / 3σ) | How close are we to the tolerance limit? Are we comfortable in the green zone, or creeping toward yellow? |
| Sigma distribution table (% within 1σ / 2σ / 3σ) | For this specific lot, how many readings were well within tolerance, how many were marginal, how many were a problem? |

## 2.2 Buffer Statistic File

Coverage: Buffer Preparation and Correction Statistic 2024–2025

This file is an aggregation of data exported by Chemical MR. It is maintained manually as a summary workbook fed by individual Chemical MR lot exports.

| Sheet | Recipe | Lots | Corrections | Notes |
| --- | --- | --- | --- | --- |
| pH 1.68 | C158 | 18 | 6 | Avg ~350 Kg/lot |
| pH 4.01 | C163 | 175 | 54 (31%) | Highest volume |
| pH 7.01 | C164 | 168 | 33 (20%) |  |
| pH 10.01 | C166 | 88 | 44 (50%) | Most critical |
| TOTAL |  | 449 | 137 | Period: Jan 2024 – Sep 2025 |

Note: corr CM [g] ← Variance (absolute deviation, grams); corr CM [%] / Corr Gibertini [%] ← Variance Perc (% deviation) from the Chemical MR per-lot export table. The exact mapping needs confirmation — see Section 7.2, Question 8.

| What i see in Excel | The question it answers |
| --- | --- |
| Production quantity per lot | How much buffer did we produce, and when? |
| FIRST QC FAILED value | When the lot failed, how far off was the pH reading from the target? |
| CM USED FOR CORRECTION | What did we add to fix it? |
| corr CM [g] / corr CM [%] | How much did we have to add |
| Corr. [%] | How far was the batch from the target recipe before any correction? |

## 2.3 Estimated Annual Volumes

| Source | Est. Volume/Year |
| --- | --- |
| Chemical QC lot files (.xlsx) | ~1,000–1,100 files |
| Chemical Production + Chemical MR PREP files (.xlsx) | ~650–700 files |
| Total files to manage | ~1,700–1,800 files/year |
| Estimated QC readings | ~120,000 new readings/year |
| Distinct product codes (sample set) | 99 Hanna Codes |

# 3. Functional Requirements

Source: Procedure Statistic Analysis QC readings — Rev. 0.0 Draft, Hanna Instruments SRL Nusfalau

## 3.1 Scope

The system manages statistics for powder reagents from the QC chemistry laboratory. Objectives: visualize lot-to-lot mean evolution trends, verify preparation precision and repeatability, and evaluate each lot quality against Hanna tolerances.

## 3.2 Key Definitions

| Term | Definition |
| --- | --- |
| STD | Reference Standard value used for calibration |
| M1–M4 | Meter 1–4: measurement instruments used during QC |
| σ | 50% of Hanna Tolerance for each STD value |
| P/Final | Test type: final production measurement |
| P/Prod | Test type: intermediate production measurement |

## 3.3 Control Chart Requirements

The main chart displays 3 overlaid series for each selected STD:

| Series | Chart Type | Description |
| --- | --- | --- |
| Raw readings | Scatter plot | All data points per lot, numbered sequentially, columns separated per lot |
| Lot-by-lot means | Scatter + line | Running average cumulative lot-by-lot |
| Sigma bands | Stacked area | 1σ green, 2σ blue, 3σ yellow (centered on STD value) |

Sigma band calculation:

- 1σ = STD Value ± (50% × Hanna Tolerance) → green

- 2σ = STD Value ± (100% × Hanna Tolerance) → blue

- 3σ = STD Value ± (150% × Hanna Tolerance) → yellow

Each STD has its own bands (tolerance varies by STD value).

## 3.4 Statistical Table Requirements

Per lot, per STD: lot number, QC date, QC week, grand total tests, total tests per STD, then count and % of readings within <±1σ, ±1σ–±2σ, ±2σ–±3σ, >±3σ.

## 3.5 Filters

- Hanna Code (product selection)

- TEST type (P/Final, P/Prod, ASTM, EPP:xxx, Old, Valid…)

- STD values: up to 6 standards per product, selectable in ascending order

- Date range

# 4. Architecture

This section describes the technologies used to build Hanna Statistics and how the data is structured internally. It is included here for completeness and for any technical review, but it does not affect the functional scope of the software.

In practical terms: the application uses a standard web technology stack — the same approach used by modern industrial software — with a dedicated database that stores all imported data in a structured, queryable format. The database runs locally on the Statistics PC in V1, with no external dependencies. If the system is later moved to a server for multi-user access (V2), the same database migrates without any changes to its structure or to the application itself.

## 4.1 Technology Stack

| Layer | Technology |
| --- | --- |
| User Interface | React 19 + TypeScript + Vite |
| UI Components | shadcn/ui + Tailwind CSS 4 |
| Charts | Apache ECharts |
| Tables | TanStack Table v8 (server-side pagination) |
| Backend / API | Node.js 20 + Express |
| Database | MariaDB 10.11+ |
| Network access | Nginx reverse proxy + HTTPS self-signed (V2 only) |
| Deployment | Docker Compose |
| Languages | EN, IT, RO, HU |

V1 (Desktop): MariaDB installed locally on Windows (Windows service, no server required). V2 (Server): same MariaDB instance migrated to Linux LAN server — no schema changes needed.

V1 / V2 — System Evolution

How the Gibertini software works today — and how Hanna Statistics will work

Chemical Production, Chemical QC and Chemical MR are installed on dedicated fixed workstations. Each application runs on one machine, each machine is in its department. Hanna Statistics will work exactly the same way in its initial version: a program installed on a PC, used by whoever needs it, with no dependency on internet connectivity or servers.

V1 — One PC, everything works Hanna Statistics is installed on a dedicated PC — typically the QC lab manager's or production manager's workstation. The .xlsx files exported by Chemical QC, Chemical MR and Chemical Production are copied to that PC (manually via USB, or automatically if a shared network folder already exists), and the software imports them, processes them and displays the results. No server, no complex network configuration. Install and use — exactly like the other Gibertini applications.

V2 — The same data, accessible from any PC on the network (optional) If in the future you wanted to access the statistics from multiple workstations — for example from both the QC lab and the production office — there would be no need to rewrite the software. It would simply be a matter of moving the database to a server already present at the plant (e.g. the existing one at 192.168.1.36) and opening a browser on any PC on the internal network. Same software, same database, same interface — accessible from multiple points simultaneously. This path is already built into the architecture proposed: it is not a hypothetical future option, it is a configuration change.

# 5. Additional Analyses

The analyses in this section go beyond what the current Excel workbooks show. They do not require any new data to be collected — everything described here is already present in the files that Chemical QC, Chemical MR and Chemical Production export today.

The current Excel setup answers one question well: how did this product perform in this lot? That is exactly what it was designed for. What it cannot do is step back and look at the bigger picture — across lots, across products, across instruments, across time.

These analyses answer a different set of questions: not what happened in this lot, but what is happening in this process. Is a particular meter slowly drifting out of calibration? Is pH 10.01 buffer always harder to get right in summer? Is one operator's preparation consistently resulting in fewer corrections downstream? Is a product that has been well within tolerance for six months starting to creep toward the limit?

None of these questions require extra work from the lab or production team. The data is already there.

The ten analyses listed below are proposed in order of value versus implementation effort. The first three — inter-meter comparison, First-Pass Yield per recipe, and early warning trend detection — deliver the highest diagnostic value for the lowest additional development cost, and are recommended for inclusion from Phase 2 onwards.

## 5.1 Reagent QC Module

### A. Inter-Meter Comparison (M1 vs M2 vs M3 vs M4)

QC files contain separate readings per meter. The official requirements aggregate them; we can show them disaggregated:

- Systematic bias per meter (e.g., M2 reads consistently +0.8 ppm above M1 → instrument review)

- Standard deviation per meter (which meter is more precise?)

- Meter degradation trend over time

Data available: M1/M2/M3/M4 columns already present in every QC .xlsx. No additional imports required.

### B. Analysis by TEST Type

Compare sigma distributions across TEST types: is P/Final more precise than P/Prod for the same lot? Which test types produce the most readings beyond 3σ?

### C. Sample Weight Analysis

The WEIGHT [mg] field is present in readings but unused in official requirements. Correlation analysis between weight variation and reading deviation from STD could reveal weighing quality issues.

### D. Intra-Lot Stability

Readings within a lot are collected across sessions. Analysis of running mean convergence: after how many readings does the mean stabilize? Do the last readings confirm the first?

### E. Early Warning (Trend Prediction)

With 25+ cumulative lots: linear regression on running average to detect drift toward ±2σ, rate of change lot-by-lot, flag "at-risk lot" if trend suggests lot N+2 may breach limits.

## 5.2 Buffer Production Module

### F. First-Pass Yield (FPY) per Recipe

Industry-standard KPI: % lots passing QC on first attempt without corrections.

| Recipe | Total Lots | No Corrections | FPY |
| --- | --- | --- | --- |
| C158 (pH 1.68) | 18 | 12 | 66.7% |
| C163 (pH 4.01) | 175 | 121 | 69.1% |
| C164 (pH 7.01) | 168 | 135 | 80.4% |
| C166 (pH 10.01) | 88 | 44 | 50.0% |

pH 10.01 is the most critical process: 1 in 2 lots requires correction. This KPI is already calculable from existing data.

### G. Seasonal Correction Analysis

pH buffers are sensitive to ambient temperature. Monthly analysis of correction rate and average CM quantity. Year-over-year comparison (Jan 2024 vs Jan 2025).

### H. Correction Material (CM) Analysis

8 distinct CMs with different roles: usage frequency, average quantity per intervention, co-occurrence in same lot, trend over time.

### I. Multi-Product Batch Analysis

The CODE column contains semicolon-separated product lists (one buffer batch feeds multiple finished products). Impact analysis of corrections on total production volume.

## 5.3 Cross-Module (QC + Buffer + PREP)

### J. Full Lot Traceability

Linking all three modules reconstructs the complete lot lifecycle: PREP batch → Buffer correction → QC result. Analyses: lead time PREP to QC Passed, correlation between buffer corrections and QC quality, operator PREP vs final quality.

## 5.4 Priority Matrix

| # | Analysis | Data Source | Effort | Value |
| --- | --- | --- | --- | --- |
| A | Inter-meter comparison | QC .xlsx | Low | High |
| F | First-Pass Yield per recipe | Buffer Statistic | Low | High |
| E | Early warning drift trend | QC .xlsx | Medium | High |
| G | Seasonal correction analysis | Buffer Statistic | Low | Medium |
| H | CM material analysis | Buffer Statistic | Low | Medium |
| B | TEST type comparison | QC .xlsx | Low | Medium |
| J | Full PREP→Buffer→QC traceability | All modules | High | High |
| C | Weight ↔ reading correlation | QC .xlsx | Medium | Medium |
| D | Intra-lot stability | QC .xlsx | Medium | Low–Med |
| I | Multi-product batch analysis | Buffer Statistic | Medium | Medium |

# 6. Import Parser Specifications

## 6.1 Chemical QC Files (Reagents)

- Metadata: rows 2–9 (lot, recipe, prep week, exp date, QC type…)

- Standards (up to 3 per file, up to 6 in system): rows 36–40

- Reading table header: row 44 (label), row 45 (columns), data from row 46+

- Meter columns: identified by position (cols 12–15), NOT by name (names vary between files: "METER 1" vs "M. 1 HI801")

- Key columns: Standard, STD_Value, TEST, QC_DATE, M1–M4, WEIGHT, QC_OPERATOR

## 6.2 Buffer Statistic File

- Multi-sheet: pH 1.68, 4.01, 7.01, 10.01 (data) + CM info (materials lookup)

- Header: row 1 = "STATISTIC PRODUCTION", col E = recipe; row 2 col E = pH value

- Column header: row 4; data from row 5

- Columns: CODE (multi-value, semicolon-separated), LOT, DATE, QUANTITY_KG, FIRST_QC_FAILED, CM_USED, CM_CODE, CORR_GIBERTINI_PCT, CORR_CM_PCT, CORR_CM_G

## 6.3 PREP Files (Preparation) — Chemical Production and Chemical MR

PREP files share a fixed layout regardless of source software. The originating software is identified from the file name pattern:

| Name Pattern | Source Software | Content |
| --- | --- | --- |
| PREP_CP-B{n}.L56.CTK.* | Chemical Production | Finished products (reagents, powders) — includes Hanna Code Table |
| PREP_B{n}-SOL.A/B.* | Chemical MR | Intermediate buffer solutions (SOL type) |
| PREP_B{n}-A/B.* | Chemical MR | Buffer component A or B |
| PREP_{HannaCode}.MR{n}.* | Chemical MR | Standard/reagent via Material Requisition |

Fixed layout (column B as data source):

- r4: Recipe, Line, Rev, Exp

- r9: Preparation Date, Batch#

- r10: Planned Week / Actual Week

- r11: Note, Operator, Exp Date

- r17+: Component Table

- (Chemical Production only): Hanna Code Table (1+ products per batch)

The parser is identical for all four patterns — only the file name classification changes.

## 6.4 Chemical MR Lot Files (Source for Module 2)

Each Chemical MR lot export contains a per-standard deviation table that feeds the Buffer Statistic aggregation file. This is the primary data source for Module 2 (Buffer Production Statistics), analogous to the Chemical QC .xlsx files for Module 1.

Expected table structure (to be confirmed with Hanna — see Section 7.2, Question 8):

| STD Number | STD Value | MR Qty | MR Acquired | Variance | Variance Perc | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Std. number | Target value | Planned qty | Actual qty | Deviation [g] | Deviation [%] |  |

Column mapping to Buffer Statistic:

- corr CM [g] ← Variance (absolute deviation in grams)

- corr CM [%] ← Variance Perc (percentage deviation)

- Corr Gibertini [%] ← likely Variance Perc (calculated by Chemical MR) — exact definition to confirm

Note: The structure of the Chemical MR per-lot export file must be confirmed with Hanna before implementing the Module 2 parser. The mapping above is inferred from the Buffer Statistic file and needs verification.

# 7. Open Technical Questions

Questions are grouped into blocking (must be resolved before development starts) and development-phase (can be resolved during implementation).

## 7.1 Blocking Questions

These are also listed in the Proposal (Section 6):

- V1 Scope: Reagent QC only, or also Buffer Production and Preparation?

- Number of workstations: 1 (standalone, local DB) or 2+ (requires data synchronization)?

- Shared network folder: accessible from production, QC lab, and Statistics PC?

- Historical import: import existing data (25 lots HI782-0, 449 buffer lots) at launch?

## 7.2 Development-Phase Questions

- TEST types: which types to support beyond P/Final and P/Prod? (ASTM, EPP, Old, Valid?)

- Users: who will use Hanna Statistics? QC lab manager only? Production manager? Both?

- Specific questions: what specific questions should the statistics answer? (e.g., "which product is most out of tolerance?", "how many buffer corrections per month?")

- Languages for V1: all four (EN/IT/RO/HU) or start with a subset?

- Hanna Core separation: should the database remain completely separate from Hanna Core?

- Data retention: how many years to keep raw readings?

- Future Hanna Core integration: is integration with Hanna Core (as a module on the same server, 192.168.1.36) a desired path?

- Chemical MR per-lot export structure (Module 2 — TO CONFIRM): Please confirm that the Chemical MR lot export files contain the table: STD Number | STD Value | MR Qty | MR Acquired | Variance | Variance Perc | Note. Also confirm the exact definition of Corr Gibertini [%] vs corr CM [%] in the Buffer Statistic file (both appear to map to Variance Perc — is one a Gibertini-calculated value and the other from Chemical MR directly?). This is required before implementing the Module 2 parser.